ETP working

1st  device will control motor1, motor2, air-blower1 and air-blower2

Alternative time motor and air-blower start and stop

Motor1 and air-blower1 will start at same time for 4 hour after that stop both motor and motor2 and air-blower2 will start this type of cycle run

2nd device will control tank1, tank2, and motor3

When etp last tank empty for first time then tank1 start mess send by 2nd device and ack will receive mess will send multiple times until ack receive and heartbeat also send every 15 sec

If 2nd board will send on mess to tank1 then power gone then tank1 check hearbeat if no hearbeat came for 10 sec then stop motor automatically

If etp last tank full then stop mess send to tank1 and motor3 will start

Again empty then tank2 will start and before starting it will check tank2 has water or not

If water there in tank2 then tank2 will start for 5 min then again cycle repeat

If tank2 not have water then tank1 will start and this cycle repeat

3rd board is there of tank1 control

It will receive mess from 2nd board and act on that mess

For dry run if motor is running for 60 min then tank1 motor will automatically off

And if no hearbeat came form 2nd board  10 sec  timeout there motor will off

4th board there to send state of all motors to database

In 4th board has 2 chips 1st  chip will collect data from all boards and send to 2nd chip using serial communication and 2nd chip have wifi and database connection

This will send all motors state to database and from web page we will see which motor is running

1st chip collect data from all boards using espnow protocoal and send to 2nd board and it will go to queue if wifi connection success then send to database using queue if wifi connection failed then all state changes will store in queue and after wifi re-connection happen all data send to supabase